Safety Data Sheet Co-Clens Hygienic Alcohol Hand Rub

Section 1: Identification of substance/preparation and of company/undertaking

1.1 Product Identifier

Trade Name: Co-Clens Hygienic Alcohol Hand Rub

Synonyms:

1.2 Relevant identified uses of the substance or mixture and uses advised against

Recommended Uses: Hand sanitiser

1.3 Details of the supplier of the safety data sheet

Supplier:

Company:	Cod Beck Blenders Ltd

Address:	Cod Beck Mill Industrial  Estate

Dalton Lane

Thirsk

YO7 3HR

Telephone:	+44 (0) 1845 577117

Email:		enquiries@cbchem.co.uk

1.4 Emergency contact number: +44(0) 1845 577117 (24hrs)

Section 2: Hazards Identification

2.1 Classification of the substance or mixture

Classification according to 1272/2008/EC:

| Physical Hazards: | Flammable Liquid 2, H225 |
| --- | --- |
| Health Hazards: | Eye irritant 2, H319 |
| Environmental Hazards: | Not classified |

Most serious harmful effects:

Eye irritation

2.2. Label elements

Labelling according to 1272/2008/EC:

GHS07		GHS02

| Signal Words | Danger |
| --- | --- |
| Hazard Statements: | H225 Highly flammable Liquid and vapour |  |
|  | H319 Causes serious eye irritation H336 May cause drowsiness or dizziness |  |
| Precautionary Statements Supplemental label information Supplementary precautionary statements | P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources no smoking P305+P351+P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses if present and easy to do. Continue rinsing P337+P313 If eye irritation persists. Get medical advice/attention P370+P378 In case of fire. Use foam, carbon dioxide, dry powder or water fog to extinguish. P403+P235 Store in well ventilated place. Keep cool. P501 Dispose of contents/container in accordance with national regulations Eye protection not required normally but wear eye protection if you are conducting an operation where there is a risk of product getting in the eyes P233 Keep container tightly closed |  |

2.3 Other hazards

This substance/mixture contains no components considered to be either persistent, bio accumulative and toxic (PBT), or very persistent and very bio accumulative (vPvB) at levels of 0.1% or higher.

Section 3: Composition/information on ingredients

3.1 Substance		Mixture

3.2 Mixture

| Name | Product identifier | Concentration limits | Classification |
| --- | --- | --- | --- |
| Ethanol | 64-17-5 | Greater than 60% | Flam. Liqu 2, Eye Irrit. 2 |
| Glycerol | 56-81-5 | Between 1 and 3 % | Not classified |
| Hydrogen peroxide | 7722-84-1 | Less than 1% | Acute Tox. 4 Skin Corr. 1A Eye Dam. 1 STOT SE 3 |

Section 4: First Aid Measures

4.1 Description of first aid measures

| Inhalation: | Remove victim to fresh air and keep at rest position comfortable for breathing |  |
| --- | --- | --- |
| Skin Contact: | In case of skin contact in excess of normal use wash and remove contaminated clothing. |  |
| Eye Contact: | Remove contact lenses if present and wash eyes with water. If irritation persists seek medical advice |  |
| Ingestion: | Rinse mouth do not induce vomiting seek medical advice |  |

4.2 Most important symptoms and effects, both acute and delayed

| Inhalation | Slight dizziness or disorientation |
| --- | --- |
| Eye contact | Mild irritation |
| Ingestion | No specific symptoms known |

| Information for Health Personnel: | No specific recommendations |
| --- | --- |

.

4.3 Indication of any immediate medical attention and special treatment needed

Specific details on antidotes:	 None known

Section 5: Firefighting Measures

| Extinguishing media: Inappropriate extinguishing media | Use foam, CO2, dry powder or water fog Avoid use of high pressure water jet which may spread the fire |  |
| --- | --- | --- |
| Special hazards arising from the substance or mixture: | Highly flammable liquid and vapour. May form explosive mixtures with air at high concentration Hazardous combustion products oxides of carbon. | . |
| Advice for firefighters: | Cool containers exposed to flames. Wear positive pressure self-contained breathing apparatus (SCBA) |  |
| Further information: | None known |  |

Section 6: Accidental Release Measures

6.1 Personal precautions, protective equipment and emergency procedures:

Personal precautions: Wear protective clothing described in section 8 in case of large spillage. Ensure that all ignition sources are removed. No smoking

6.2 Environmental precautions Environmental precautionary measures:

.Do not discharge into drains or watercourses or onto ground. Contain spillage with inert absorbent material

6.3 Methods and material for containment and cleaning up:

Eliminate all sources of ignition, provide adequate ventilation absorb onto inert material such as sand or vermiculite and place into container. Wash after dealing with spillage

6.4 Reference to other sections:

For personal protection see section 8

Section 7: Handling and storage

7.1 Precautions for safe handling:

Store in a well ventilated area away from heat, sparks or open flame. No smoking

7.2 Protective Safety Measures:

Store bulk supplies in flammable liquids storage

7.3 Advice on general occupational hygiene:

This material is intended as a hand sanitiser.

7.4 Conditions for safe storage, including any incompatibilities:

Store in tightly closed contain ensure that the container is closed after use.

7.5 Conditions to Avoid:

Avoid ignition sources and elevated temperatures

7.6 Specific end use(s):

This product is a hand sanitiser for general use

Section 8: Exposure controls/personal protection

8.1. Control parameters:

Occupational exposure limits

Ethanol

Long term exposure limit (WEL) (8hr TWA) 1000ppm, 1920mg/m3

Glycerine

Long term exposure limit (8hr TWA) 10mg/m3 as mist

8.2 Exposure Controls / Occupational Exposure Limits:

Safety signs

| Engineering controls | Not relevant |
| --- | --- |
| Respiratory Protection: | Not normally required. In case of a large spillage wear full face mask with organic fumes filters |
| Hand protection: | Not normally required |
| Eye / face protection: | Not normally required but wear eye protection if you are conducting an operation where there is a risk of eye contact |
| Skin protection: | Not required |

Section 9: Physical and Chemical Properties

9.1 Information on basic physical and chemical properties:

| Appearance: | Gel |
| --- | --- |
| Colour: | Colourless to cream |
| Odour: pH: Density | Alcohol N/a Not determined |
| Flash Point: | Not determined classification based on ethanol content |
| Flammability Range: | Not determined |
| Miscibility with Water: | Miscible |

9.2 Other safety information:

None

Section 10: Stability and reactivity

| 10.1 | Reactivity: | There are no known reactivity hazards associated with this product |
| --- | --- | --- |
| 10.2 | Chemical Stability: | No stability concerns |
| 10.3 | Possibility of Hazardous Reactions: | None determined |
| 10.4 | Conditions to Avoid: | Avoid ignition sources and high temperatures |
| 10.5 | Incompatible Materials: | None known |
| 10.6 | Hazardous Decomposition Products: | Oxides of carbon |
| Section 11: Toxicological Information |  |  |

11.1 Information on Toxicological Effects:

| General Information: | This product is a hand sanitiser and when used as directed is not expected to be a hazard to health. The following data for ethanol is indicative only |
| --- | --- |
| Inhalation: | Species Rat ATE inhalation (vapours mg/l) 124.7 |
| Ingestion: | LD₅₀ 10470 mg/kg, Oral, Rat |
| Skin Contact: | Skin - Rabbit Result: No skin irritation (OECD Test Guideline 404) Acute toxicity - dermal Notes (dermal LD₅₀) LD₅₀ >2000 mg/kg, Dermal, Rabbit |
| Eye Contact: Respiratory sensitisation Skin sensitisation Germ cell Carcinogenicity Carcinogenicity Mouse Reproductive toxicity Specific target organ toxicity Specific target organ toxicity | Eyes - Rabbit Result: Eye irritation Based on available data the classification criteria are not met. Skin sensitisation - Guinea pig: Not sensitising. Mutagenicity Genotoxicity - in vitro Bacterial reverse mutation test: Negative. Genotoxicity - in vivo Chromosome aberration: Negative. NOAEL > 4250 mg/kg/day, Oral Reproductive toxicity - fertility Based on available data the classification criteria are not met. Single exposure STOT - single exposure Based on available data the classification criteria is not met. repeated exposure STOT - repeated exposure LOAEL 3160 mg/kg, Oral, Rat Target organs Kidneys |
| Acute and Chronic Health Hazards: |  |
| Medical Symptoms: | Eye irritation, respiratory irritation, nausea dizziness and intoxication if ingested or inhaled |
| Medical Considerations: | Remove from exposure and treat symptomatically |

Section 12: Ecological Information

This product is not expected to be hazardous to the environment. The data below is indicative based on ethanol

| 12.1 | Toxicity: | Toxicity to fish LC₅₀, 96 hours: 11,200 mg/l, Freshwater fish Toxicity to daphnia and other aquatic invertebrates LC₅₀, 48 hours: 12340 mg/l, Daphnia magna Toxicity to plants EC₅₀, 72 hours: 12,900 mg/l, Selenastrum capricornutu |
| --- | --- | --- |
| 12.2 | Persistence and Degradability: | Water - Degradation (%) 75: 20 days The substance is readily biodegradable. it.) |
| 12.3 | Bio Accumulative Potential: | No bioaccumulation is to be expected (log Pow <= 4). |
| 12.4 | Mobility in Soil: | No data available |
| 12.5 | Results of PBT and vPvB Assessment: | This substance/mixture contains no components considered to be either persistent, bio accumulative and toxic (PBT), or very persistent and very bio accumulative (vPvB) at levels of 0.1% or higher. |

Section 13: Disposal considerations

.

13.1 Waste treatment methods:

| General Information | Dispose of to licensed waste disposal company within the requirements of local regulation |
| --- | --- |
| Disposal Method | Reuse or recycle when possible. Disposal in accordance with local regulation |

Section 14: Transport information

General:

| 14.1 | UN Number: |  | UN 1987 |
| --- | --- | --- | --- |
| 14.2 | UN Proper Shipping Name: |  | Alcohols NOS (Propanol) |
| 14.3 | Transport Hazard Class(es): |  | 3 |
| 14.4 | Packing Group: |  | II |
| 14.5 | Environmental Hazards: |  | Environmentally hazardous substance/marine pollutant: |
| 14.6 | Special Precautions for User: |  |  |
| Emergency Action code |  | .3YE |  |
| Hazard Identification Number (ADR/RID) |  | 33 |  |
| Tunnel restriction code |  | (D/E) |  |

Section 15: Regulatory information

15.1 Safety, health and environmental regulations/legislation specific for the substance or mixture:

| EU legislation | Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures (as amended). Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH) (as amended). |
| --- | --- |
| Guidance | Workplace Exposure Limits EH40. |

Section 16: Other information

Key literature references and sources for data: European Chemicals Agency, http://echa.europa.eu/

The mixture should not be used for purposes other than those described in section 1 of this document

Issued by: Job title required

Basis of classification: Calculation from formulation

Revision date:  0 Revision 0