Safety Data Sheet

Section 1: Identification of substance/mixture and of company/undertaking

1.1 Product Identifier            Chlorhexidine 2% in 70% Ethanol (clear)

Trade Name:

Synonyms:

1.2 Relevant identified uses of the substance or mixture and uses advised against

Recommended Uses: Professional skin antiseptic

1.3 Details of the supplier of the safety data sheet

Supplier:

Company:	Co-Clens Ltd

Address:	Cod Beck Mill Industrial Estate

Dalton Lane

Thirsk

YO7 3HR

Telephone:	+44 (0) 1845 577117

Email:		enquiries@cbchem.co.uk

1.4 Emergency telephone number: +44(0) 1845 577117 (24hrs)

Section 2: Hazards identification

2.1 Classification of the substance or mixture

Classification according to 1272/2008/EC:

| Physical Hazards: | Flammable Liquid 2, H225 |
| --- | --- |
| Health Hazards: | Eye Irritant, H319 |
| Environmental Hazards: | Acute Chronic 3, H412 |

Most serious harmful effects:

The product is highly flammable

The product may cause eye irritation

The product is harmful to aquatic life with long lasting effect

2.2. Label elements

Labelling according to 1272/2008/EC:

Signal Words

Danger

Hazard Statements:

| H225 | Highly Flammable Liquid and vapour |
| --- | --- |
| H319 | Causes serious eye irritation |
| H412 | Harmful to aquatic life with long-lasting effects |

Precautionary Statements:

| P210 | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| --- | --- |
| P233 | Keep container tightly closed |
| P305, P351, & P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses if present and easy to do so. Continue rinsing |
| P337 & P313 | If eye irritation persist, get medical advice/attention. |
| P403 & P235 | Store in well ventilated place. Keep cool |
| P501 | Dispose of contents/container in accordance with national regulations |

Supplementary Label Information

Supplementary precautionary statements

2.3 Other hazards

None.

Section 3: Composition/information on ingredients

This product is a mixture

3.2 Mixtures

| Name | Numerical identifiers |  | Concentration limits | Classification |
| --- | --- | --- | --- | --- |
| Ethanol | CAS EINECS REACH | 64-17-5 200-578-6 01-2119457610-43 | >67% - <69% | Flammable Liquid 2; H225 Eye irritation 2; H319 |
| Chlorhexidine gluconate | CAS EINECS REACH | 18472-51-0 242-354-0 01-2119946568 | >1.8%-<2.5% | Eye Damage 1; H318 Aquatic Acute 1 (M-Factor =1); H400 Aquatic Chronic 1 (M-Factor = 1); H410 |

Section 4: First aid measures

4.1 Description of first aid measures

| Inhalation: | Remove victim to fresh air and keep at rest position comfortable for breathing |
| --- | --- |
| Skin Contact: | In case of skin contact in excess of normal use, wash and remove contaminated clothing |
| Eye Contact: | Remove contact lenses if present and wash eyes with water. If irritation persists seek medical advice. |
| Ingestion: | Rinse mouth. Do not induce vomiting. Seek medical advice. |

4.2 Most important symptoms and effects, both acute and delayed

| Inhalation | Disorientation |
| --- | --- |
| Eye contact | Irritation |
| Ingestion | Disorientation and nausea |

| Information for Health Personnel: | No specific recommendations |
| --- | --- |

4.3 Indication of any immediate medical attention and special treatment needed

Treat symptomatically

Specific details on antidotes:	Not required

| Section 5: Firefighting measures |  |  |
| --- | --- | --- |
| 5.1 Extinguishing media Inappropriate extinguishing media | Use extinguishing measures that are appropriate to local circumstances and the surrounding environment. Powder alcohol resistant foam and water spray High pressure water jet |  |
| 5.2 Special hazards arising from the substance or mixture | Flash back possible over considerable distance. Beware of vapours accumulating to form explosive concentrations. Vapours can accumulate in low areas. Decomposition products may include the following materials: Carbon oxides nitrogen oxides (NOx) | . |
| 5.3 Advice for firefighters: | Wear self- contained breathing apparatus Use water spray to cool unopened containers. Fire residues and contaminated fire extinguishing water must be disposed of in accordance with local regulations. |  |

Section 6: Accidental release measures

6.1 Personal precautions, protective equipment and emergency procedures:

Remove all sources of ignition. Wear full cover eye protection; goggles or face shield, gloves; nitrile, PVC or butyl, a water proof coverall.

6.2 Environmental precautions

Do not discharge into drains or watercourses or onto ground. Contain spillage with inert absorbent material.

6.3 Methods and material for containment and cleaning up:

Contain large spillages with booms or dry absorbent material and use absorbent to collect spillage. Transfer to containers for disposal by a licenced waste disposal contractor.

Wash area thoroughly do not allow washings to enter watercourses or spread onto unmade ground. Very dilute washings may be collected and pumped to containers for disposal or allowed to go to mains sewers. Do not allow washings to enter storm drains

Small spillages may be collected on absorbent material for disposal in a ventilated container or diluted and washed to sewer.

6.4 Reference to other sections:

For personal protection see section 8

Section 7: Handling and storage

7.1 Precautions for safe handling

Use only with adequate ventilation. Keep away from fire, sparks and heated surfaces. Take necessary action to avoid static electricity discharge (which might cause ignition of organic vapours)

Handle in accordance with good hygiene and safety practice. Remove and wash contaminated clothing before re-use.

7.2 Conditions for safe storage, including any incompatibilities

Keep away from heat and sources of ignition. Keep in a cool, well ventilated place. Keep away from oxidizing agents. Keep out of reach of children. Keep container tightly closed. Store in suitable labelled containers.

Storage conditions-0 °C to 25 °C

7.3 Specific end use(s)

Antiseptic

7.4 Conditions to Avoid High temperatures

Section 8: Exposure controls/personal protection

8.1. Control parameters

Occupational exposure limits

Ethanol

Long term exposure limit (WEL) (8hr TWA)  		1000ppm/1920mg/m3

Short term exposure limit (STEL:			No limits established

8.2 Exposure Controls

Safety signs

| Engineering controls: | Not applicable |
| --- | --- |
| Respiratory Protection: | Not applicable in normal use. In case of a major spillage wear a respirator fitted with the following cartridge: Organic vapour filter. Check that the respirator fits tightly and the filter is changed regularly. |
| Hand protection: | For repeat contact wear nitrile gloves |
| Eye / face protection: | Wear safety glasses or face shield if splashing could occur |
| Skin protection: | Not applicable. |

Section 9: Physical and chemical properties

9.1 Information on basic physical and chemical properties

| Appearance: | Liquid |
| --- | --- |
| Colour: | Clear |
| Odour: pH: Relative density | Alcoholic 6-8 0.8-0.9 |
| Flash Point: | 20oC Closed cup |
| Flammability Range: | Upper flammable/explosive limit: 19 % Lower flammable/explosive limit: 3.5 % (ethanol) |
| Miscibility with Water: | Fully soluble |

9.2 Other information: None

Section 10: Stability and reactivity

| 10.1 | Reactivity | No dangerous reactions under normal storage and use conditions |
| --- | --- | --- |
| 10.2 | Chemical stability | Stable under normal conditions |
| 10.3 | Possibility of hazardous reactions | None known |
| 10.4 | Conditions to avoid | Heat, flames, sparks |
| 10.5 | Incompatible materials | None known |
| 10.6 | Hazardous decomposition products | Oxides of carbon and nitrogen |

11.0 Information on toxicological effects

11.1 General Information

| Information on likely routes of exposure | Inhalation, Eye contact, Skin contact. |
| --- | --- |

| Inhalation: | Acute inhalation toxicity: | Ethanol 4 h LC50 rat: 117 mg/l |
| --- | --- | --- |
| Ingestion: | Acute oral toxicity: | Ethanol LD50 rat: 10,470 mg/kg Chlorhexidine gluconate LD50 rat: > 2,000 mg/kg |
| Skin Contact: | Acute dermal toxicity: | Ethanol LD50 rabbit: > 2,000 mg/kg Chlorhexidine gluconate LD50 rat: > 5,000 mg/kg |
|  | Skin corrosion/irritation: | Result: No skin irritation - 24 h (OECD Test Guideline 404) |
| Eye Contact: Irritation/Damage |  | ethanol Rabbit Result: Causes serious eye irritation. (OECD Test Guideline 405) |
| Respiratory or skin sensitization Germ cell mutagenicity Carcinogenicity Reproductive toxicity Specific target organ toxicity - single exposure Acute inhalation toxicity - Specific target organ toxicity - repeated exposure Aspiration hazard |  | No data available No data available IARC: No ingredient of this product presents at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC. No data available No data available Possible symptoms:, mucosal irritations No data available No data available |
| Acute and Chronic Health Hazards: |  | Under normal use no effects are expected. Excessive exposure to vapour or ingestion could cause; dizziness and narcosis. Direct contact could cause short term eye irritation. |
| Medical Symptoms: |  | ethanol irritant effects, respiratory paralysis, Dizziness, narcosis, inebriation, euphoria, Nausea, Vomiting |
| Medical Considerations: |  | Treat symptomatically remove from exposure |

| Section 12: Ecological information |  |  |
| --- | --- | --- |
| 12.1 | Toxicity |  |
|  | Toxicity to fish Toxicity to daphnia and other aquatic invertebrates Toxicity to algae Toxicity to bacteria | Ethanol: flow-through test LC50 - Pimephales promelas (fathead minnow) -15,300 mg/l - 96 h (US-EPA) static test LC50 - Ceriodaphnia dubia (water flea) - 5,012 mg/l - 48h Remarks: (ECHA) static test ErC50 - Chlorella vulgaris (Fresh water algae) - 275 mg/l - 72 h (OECD Test Guideline 201) static test LC50 - activated sludge - > 1,000 mg/l - 3 h (OECD Test Guideline 209) |
|  | Toxicity to fish Toxicity to daphnia and other aquatic invertebrates Toxicity to algae Toxicity to bacteria | Chlorhexidine: LC50, 96 hours: 2.08 mg/l. Fish EC50 – Daphnia magna – 0.087 mg/l -48 hours ErC50 – Green algae – 0.081 mg/l – 72 hours ErC50 – Activated sludge – 25 mg/l - 3 hours |
| 12.2 | Persistence and degradability |  |
|  | Biodegradability aerobic - Exposure time 15 d Biochemical Oxygen Demand (BOD) Theoretical oxygen demand Biodegradability aerobic - Exposure time 10 d | Ethanol : ca.95 % - Readily biodegradable. (OECD Test Guideline 301E) 930 - 1,670 mg/g Remarks: (Lit.) 2,100 mg/g Remarks: (Lit.) Chlorhexidine >70% - Readily biodegradable. |
| 12.3 | Bio accumulative potential | Due to the distribution coefficient n-octanol/water, accumulation in organisms is not expected. |
| 12.4 | Mobility in soil | No data available |
| 12.5 | Results of PBT and vPvB assessment | This substance/mixture contains no components considered to be either persistent, bio accumulative and toxic (PBT), or very persistent and very bio accumulative (vPvB) at levels of 0.1% or higher. |
| 12.6 | Other adverse effects | None known |
| Section 13: Disposal considerations |  |  |

13.1 Waste treatment methods:

| General Information | Dispose of in accordance with the national regulations on waste and hazardous waste |
| --- | --- |
| Disposal Method | . |

Product: Where possible recycling is preferred to disposal or incineration. If recycling is not practicable, dispose of in compliance with local regulations. Dispose of wastes in an approved waste disposal facility.

Contaminated packaging: Dispose of as unused product. Empty containers should be taken to an approved waste handling site for recycling or disposal. Do not re-use empty containers.

European Waste Catalogue: 200113* - solvents

Section 14: Transport information

Land transport (ADR/ADN/RID)

| 14.1 | UN number |  | 1170 |
| --- | --- | --- | --- |
| 14.2 | UN proper shipping name |  | Ethanol Solution |
| 14.3 | Transport hazard class(es) |  | 3 |
| 14.4 | Packing group |  | II |
| 14.5 | Environmental hazards |  | No |
| 14.6 | Special precautions for user |  | No |
| Air transport (IATA) |  |  |  |
| 14.1 | UN number |  | 1170 |
| 14.2 | UN proper shipping name |  | Ethanol Solution |
| 14.3 | Transport hazard class(es) |  | 3 |
| 14.4 | Packing group |  | II |
| 14.5 | Environmental hazards |  | No |
| Sea transport (IMDG/IMO) |  |  |  |
| 14.1 | UN number |  | 1170 |
| 14.2 | UN proper shipping name |  | Ethanol Solution |
| 14.3 | Transport hazard class(es) |  | 3 |
| 14.4 | Packing group |  | II |
| 14.5 | Environmental hazards |  | No |
| 14.6 | Special precautions for user |  | None |
| 14.7 | Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code |  | Not applicable |

Section 15: Regulatory information

15.1 Safety, health and environmental regulations/legislation specific for the substance or mixture:

| EU legislation | Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures (as amended). Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH) (as amended). |
| --- | --- |
| Workplace exposure limits | UK: EH40/2005 (Third edition; 2018) Ireland: Code of Practice for the Chemical Agents Regulations (2018) |

15.2 Chemical Safety Assessment

This product contains substances for which Chemical Safety Assessments are still required.

Section 16: Other information

Safety Statements:

| H225 | Highly flammable liquid and vapour. |
| --- | --- |
| H319 | Causes serious eye irritation |
| H400 | Very toxic to aquatic life. |

Precautionary Statements:

# Abbreviations used in this Safety Data Sheet:

| # IBC code | # International code for the construction and equipment of ships carrying dangerous chemicals in bulk |
| --- | --- |
| # LC50 | # Lethal concentration required to kill 50% of test population |
| # LD50 | # Lethal dose at which 50% of test population is killed within a certain time |
| # MARPOL | # International convention for the prevention of pollution from ships (1973) as modified (1978) |
| # PBT | # Persistent, bioaccumulative and toxic |
| # REACH | # Registration, Evaluation. Authorisation and Restriction of Chemicals (Regulation (EC) No 1907/2006) |
| # UN | # United Nations Model Regulations on the Transport of Dangerous Goods |
| # vPvB | # Very persistent and very bioaccumulative |

Key literature references and sources for data: European Chemicals Agency, http://echa.europa.eu/

The mixture should not be used for purposes other than those described in section 1 of this document

Issued by: Health & Safety Advisor

Basis of classification: Regulation (EC) No 1272/2008 (as amended) plus associated guidance

Changes made in this Safety Data Sheet: None (original version)